[image: image1.png]& Fermiab —‘L-——!




Fermi National Accelerator Laboratory

General Information for Manufacture of Printed Circuit Boards

Note: All dimensions in inches unless specified

	TOF CLOCK TRANSLATOR
	Board Name

	10
	Quantity

	2 signal, 2 power
	Board Type (# signal layers - # power layers)

	Not less than 63 mils
	Board Thickness and Tolerance

	7.125 x 9.000 inches
	Board Dimensions and shape 

	SMOBC
	Finish       SMOBC  (solder mask over bare copper) 

	HASL
	Plating     HASL      (hot air solder leveling)

	1
	Cu Weight outer layers (ounces per square inch)

	1
	Cu Weight inner layers (ounces per square inch)

	None
	Number of gold contacts/side

	10 mils
	Minimum Trace width & tolerance

	6 mils
	Minimum Trace clearance & tolerance

	Top and bottom
	Solder Mask ( top & bottom – top only)

	lpi
	Solder Mask Type (wet/dry/LPI)

	Top and bottom
	Silk Screen ( top & bottom – top only - white)

	yes
	Testing Required (yes/no/specify)

	~75 top and ~175 bottom
	Number of Surface mount pads TOP/BOT (only needed for testing)

	248
	Number of Through Holes 

	7
	Number of Drill Sizes

	13 mils
	Smallest Hole

	130 mils
	Largest Hole

	None
	Number of Blind Vias

	None
	Number of Buried Vias

	100 ohm differential traces on bottom layer: 16 mil width, 6 mil separation.
	Controlled Impedance (yes/no/specify)

	---------------------------------
	Physical Board Stackup

	TOP
	
Top layer name

	POWER
	
Inner layer

	GND
	
Inner layer

	BOTTOM
	
Bottom layer name

	Gerber files
	Material Supplied (Film/gerber files/modem/ect)

	5/10 days
	Required Delivery

	
	Special NOTES 

	Matthew Jones – 630-840-8410
Tom Wesson – 630-840-2948
	Technical Contact Personnel and phone number


