### CERTIFICATION OF HAZARD ASSESSMENT

| Supervisor (print): George S. Jackson | Assessment Date(s): 05/30/2012 |
| --- | --- |
| Signature: | Location(s) posted: B24 |

| # Hazards | Task: hands-on work or being within reach(a) of potential hazards of described activity/items: | Minimum Requirements |
| --- | --- | --- |
| Skin/eye damage burns | Minimum PPE for any work in room other than desk work | Shower and eyewash must be available and functional in work area. Skin cover to throat/wrists/ankles closed shoes, safety glasses, and chemical resistant gloves(h). Add a lab coat if chemicals are in use. |
| Skin/eye damage, poisoning, inhalation of vapor or aerosol | Volume > 10 mL any unshielded(b) corrosive(c) liquids, organic liquids or liquid mixtures, or toxic(d) inorganic liquids/mixtures | Splash goggles, chemical resistant gloves(e), lab coat, skin cover to knees/elbows/throat, closed shoes with socks. Work in hood(f). Shower and eyewash must be available in work area. |
|  | Volume > 1 L | Same, but cover to ankles/wrists/throat |
| Cataracts, flash burns to cornea | Work with infrared emitting equipment (glass and quartz blowing) | Appropriate shaded goggles Lab coat, closed shoe, pants |
| Skin/eye damage | Hot liquid (acid dissolution of biological samples for 26Al analysis) | Splash goggles, insulated gloves, skin cover to knees/elbows/throat, closed shoe w/ socks |
| Cell damage, area contamination | Radioactive materials | Shielding and badging requirements prescribed in specific isotope SOP, use all appropriate chemical and/or biological safety personal protection |

## NOTES

(a)	Being within reach of potential hazards: "within reach" varies widely depending on scale and conditions of work and will be judged by affected staff in each room.

(b)	Unshielded: not behind a drawn hood sash or blast shield.

(c)	Corrosive: pH  12 or pH  2.5

(d)	Toxic: having any poisonous or irritating effects to human tissue or human health.

(e)	Chemical resistant gloves: glove thickness, length, and material must be chosen carefully and will be specific to the chemicals/mixtures used and the process conditions.

(f)	Hood: 100% exhaust to outside, current approval for "all work" and functioning properly.

(g)	Chemicals requiring designated areas: full list is at http://www.purdue.edu//rem/home/booklets/crdalist.pdf

(h) Chemical resistant gloves: glove thickness, length, and material must be chosen carefull

and will be specific to the chemicals/mixtures used in the process conditions.