### CERTIFICATION OF HAZARD ASSESSMENT

#### INTRODUCTION

"Hazard assessment" is the process (required by law) of identifying the hazards associated with defined task, prescribing personal protective equipment and other relevant protection measures which must be employed to reduce the risk from the hazards.  "Certification of Hazard Assessment" is a written document -- such as the one on the following 2 pages -- detailing the hazard assessment(s) for (a) particular task(s).  The supervisor is responsible for ensuring that hazard assessments are performed and the certification(s) written and posted.  The supervisor may delegate or contract the labor involved in this process, but cannot reassign or disclaim the responsibility.

###### INSTRUCTIONS

- Save the attached hazard assessment example to your hard drive.

- It must be modified to meet the specific hazards of your work area.  This includes removing or adding hazards as applicable to your work area.

- Certification(s) of hazard assessments must be posted -- tacked or hung in a visible place -- in every work room listed in the "location(s)" field.

- The fields at the beginning -- date(s), location(s), supervisor, and signature -- must be completed.

Post signed certification in work room.

| Supervisor (print): | Assessment Date(s): |
| --- | --- |
| Signature: | Location(s) posted: |

| # Hazards | Task: hands-on work or being within reach(a) of potential hazards of described activity/items: | Minimum Requirements |
| --- | --- | --- |
| Skin/eye damage, poisoning, inhalation of vapor or aerosol | Volume > 10 mL any unshielded(b) corrosive(c) liquids, organic liquids or liquid mixtures, or toxic(d) inorganic liquids/mixtures | Splash goggles, chemical resistant gloves(e), lab coat, skin cover to knees/elbows/throat, closed shoes with socks. Work in hood(f). Shower and eyewash must be available in work area. |
|  | Volume > 1 L | Same, but cover to ankles/wrists/throat |
|  | Volume > 5 L | Add face shield covering chin |
| Cataracts, flash burns to cornea | Work with infrared emitting equipment (glass blowing) | Appropriate shaded goggles Lab coat, closed shoe, pants |
| Conjunctivitis, corneal damage, erythema | Arc/TIG welding | Appropriate shaded goggles Working gloves |
| Skin/limb injury | Machine operation activities likely to catch clothing, hair, or jewelry | Bind vulnerable clothing/hair, remove jewelry |
| Eye impact | Metalworking, woodworking, other operations likely to throw particles | Safety glasses No loose clothing or jewelry |
| Head impact | Working or walking in area having potential of falling tools, equipment, or stored items | Hard hat |
| Skin/eye damage | Cryogenic liquids | Splash goggles, skin cover to elbows/knees/throat, closed shoe easily removed, socks. Cryogloves for dispensing. |
|  | Volume > 1 L | Skin cover to throat/wrists/ankles |
| Skin/eye damage, asphyxiation, body injury, frostbite, | Transport of liquid nitrogen in hallways and elevators | See cryogenic liquids; also all wheeled vessels or carts must restrain Dewar and have wheels large enough to safely traverse elevator door and scales gap. |
|  | Self pressurizing vessels weighing > 100 lb gross | Plus skin cover to wrists/throat/ankles, always position blow-off valve away from body |
|  | Dispensing from main bulk tank | All above and hearing protection |
| Frostbite, eye impact | Dry ice, very cold frozen solids. | Safety glasses, insulated gloves, skin cover to elbows/knees/throat, closed shoe w/ socks |
| Skin/eye damage | Hot liquid (rxn mixture, water bath, oil bath, autoclave, still...) | Splash goggles, insulated gloves, skin cover to knees/elbows/throat, closed shoe w/ socks |
|  | Volume > 1 L | Skin cover to throat/wrists/ankles, emergency shower available in work area |
| Eye damage, Erythema | UV radiation | UV blocking goggles, skin cover on all potentially exposed areas |
|  | Potential face UV exposure | UV face shield |

| Hazards | Task: (hands-on work or being within reach(a) of potential hazards of described activity/items | Minimum Requirements |
| --- | --- | --- |
| Skin/eye damage | Laser radiation | Goggles appropriate to beam parameters, closed shoe, no jewelry/reflective items |
|  | Class 3b and 4 lasers | Skin cover on all potentially exposed areas |
| Infectious disease | Human blood, cells, tissue, body fluids or materials derived from same | Safety glasses, "exam" gloves, skin cover on all potentially exposed areas, shoes/socks, work at Biosafety Level II. |
|  | Liquid with vol > 1 mL | Same, but splash goggles, skin cover to throat/wrists/ankles |
| Skin/eye damage, poisoning, inhalation of airborne dust | Hazardous solids | Safety glasses, goggles for large quantities, chemical resistant gloves, skin cover to elbows/knees/throat, closed shoes/socks |
|  | > 100 g any hazardous solid, or > 1 g "chemical requiring designated area," (list at REM web site(g)) or High potential for airborne particles | Same, except skin cover to wrists/ankles, and only work in hood |
| Cell damage, area contamination | Radioactive materials | Shielding and badging requirements prescribed in specific isotope SOP, use all appropriate chemical and/or biological safety personal protection |

## NOTES

(a)	Being within reach of potential hazards: "within reach" varies widely depending on scale and conditions of work and will be judged by affected staff in each room.

(b)	Unshielded: not behind a drawn hood sash or blast shield.

(c)	Corrosive: pH  12 or pH  2.5

(d)	Toxic: having any poisonous or irritating effects to human tissue or human health.

(e)	Chemical resistant gloves: glove thickness, length, and material must be chosen carefully and will be specific to the chemicals/mixtures used and the process conditions.

(f)	Hood: 100% exhaust to outside, current approval for "all work" and functioning properly.

(g)	Chemicals requiring designated areas: full list is at http://www.purdue.edu//rem/home/booklets/crdalist.pdf