| Supervisor (print): Greg Chmiel | Assessment Date(s): 2019-05-22 |
| --- | --- |
| Signature: | Location(s) posted: Wetherill 206 |

Note: Combining tasks combines the Minimum Requirements

| # Task (hands-on work or being within reach(a) of potential hazards of described activity/items): | Hazards: | Minimum Requirements: |
| --- | --- | --- |
| # Being present in the laboratory | As per the Chemical Hygiene Plan | Door must be unlocked, safety glasses on, nitrile gloves on, lab coat or chemical-resistant apron on, long pants and closed-toed shoes with socks, shower and eyewash must be in working order |
| # Working alone in the laboratory | See below and on the HF-specific Certification of Hazard Assessment | Following is the list of people who are allowed to work alone in Wetherill 206; all others WILL NOT work unsupervised: Greg Chmiel, Darryl Granger, Angus Moore, Jane Andersen |
| # Concentrated Acids/Bases # All work with concentrated acids—68% Nitric, 37% Hydrochloric, 98% Sulfuric—and bases—50% Sodium and 30% Ammonium hydroxides | Skin/eye damage; poisoning, inhalation of corrosive vapor or aerosol | All work must be done in a hood(f); have water running in a sink external to the hood (to wash gloves), safety glasses, nitrile gloves (but switch to chemical resistant gloves(e) for work taking an extended time), chemical-resistant apron, PVC arm shields Volume > 1 L: Add chemical-resistant overboots and coat apron Volume > 5 L: Add face shield covering chin |
| # Corrosive Liquids # Working with unshielded(b) corrosive(c) liquids—but not concentrated acids/bases | Skin/eye damage; poisoning, inhalation of corrosive vapor or aerosol | Safety glasses, nitrile gloves with PVC arm shields OR extended-cuff nitrile gloves, chemical-resistant apron but: Volume < 10 mL: Minimum requirements in Line 1 for any/all work in laboratory apply Shielded liquid (in capped centrifuge tube): Minimum requirements for any/all work in laboratory apply |
| # Organic Liquids # Working with Acetone, Methanol, Ethanol | Skin/eye damage; poisoning, inhalation of corrosive vapor or aerosol | All working volumes when using organic liquids are < 10 mL Minimum requirements for any/all work in laboratory apply But for Acetone—squeeze bottle use—additionally all work must be done in a hood(f) |

| # Task (hands-on work or being within reach(a) of potential hazards of described activity/items): | Hazards: | Minimum Requirements: |
| --- | --- | --- |
| # Hot Liquids # Working with hot liquid samples on hotplates | Skin/eye damage | All work must be done in a hood(f); safety glasses, nitrile gloves, chemical-resistant apron, PVC arm shields Volume > 1 L: Add chemical-resistant overboots and coat apron |
| # Hazardous Liquids # Working with chemicals requiring designated areas(g) # Methylene Choloride / Dichloromethane | Causes skin irritation, causes serious eye irritation, may cause drowsiness or dizziness, may cause cancer | Safety glasses, Polyvinyl alcohol (PVA) gloves with PVC arm shields, chemical-resistant apron |
| # Accessing/Working at heights greater than eye level | Fall | Use portable ladder in the lab to reach shelves or perform other tasks that are higher than eye-level. Do NOT stand on chairs, stools or counters for these tasks. |

(a)	Being within reach of potential hazards: For Wetherill 208 "within reach" is defined as five (5) feet.

(b)	Unshielded: not behind a drawn hood sash or blast shield.

(c)	Corrosive: pH  12 or pH  2.5

(d)	Toxic: having any poisonous or irritating effects to human tissue or human health.

(e)	Chemical resistant gloves: glove thickness, length, and material must be chosen carefully and will be specific to the chemicals/mixtures used and the process conditions.

(f)	Hood: 100% exhaust to outside, current approval for "all work" and functioning properly.

(g)	Chemicals requiring designated areas: full list is at https://www.purdue.edu/ehps/rem/documents/programs/crdalist.pdf