| Supervisor (print): Greg Chmiel | Assessment Date(s): 2019-05-22 |
| --- | --- |
| Signature: | Location(s) posted: Wetherill 208 |

Note: Combining tasks combines the Minimum Requirements

| # Task (hands-on work or being within reach(a) of potential hazards of described activity/items): | Hazards: | Minimum Requirements: |
| --- | --- | --- |
| # Being present in the laboratory | As per the Chemical Hygiene Plan | Door must be unlocked, safety glasses on, nitrile gloves on, lab coat or chemical-resistant apron on, long pants and closed-toed shoes with socks, shower and eyewash must be in working order |
| # Working alone in the laboratory | See below and on the HF-specific Certification of Hazard Assessment | Following is the list of people who are allowed to work alone in Wetherill 208; all others WILL NOT work unsupervised: Greg Chmiel, Darryl Granger, Tom Woodruff, Angus Moore, Jane Andersen |
| # Concentrated Acids/Bases # All work with concentrated acids—68% Nitric, 37% Hydrochloric, 98% Sulfuric—and bases—50% Sodium and 30% Ammonium hydroxides | Skin/eye damage; poisoning, inhalation of corrosive vapor or aerosol | All work must be done in a hood(f); have water running in a sink external to the hood (to wash gloves), safety glasses, nitrile gloves (but switch to chemical resistant gloves(e) for work taking an extended time), chemical-resistant apron, PVC arm shields Volume > 1 L: Add chemical-resistant overboots and coat apron Volume > 5 L: Add face shield covering chin |
| # Corrosive Liquids # Working with unshielded(b) corrosive(c) liquids—but not concentrated acids/bases | Skin/eye damage; poisoning, inhalation of corrosive vapor or aerosol | Safety glasses, nitrile gloves with PVC arm shields OR extended-cuff nitrile gloves, chemical-resistant apron but: Volume < 10 mL: Minimum requirements in Line 1 for any/all work in laboratory apply Shielded liquid (in capped centrifuge tube): Minimum requirements for any/all work in laboratory apply |
| # Organic Liquids # Working with Acetone, Methanol, Ethanol, and (3-Aminopropyl)triethoxysilane | Skin/eye damage; poisoning, inhalation of corrosive vapor or aerosol | All working volumes when using organic liquids are < 10 mL Minimum requirements for any/all work in laboratory apply But for Acetone—squeeze bottle use—and (3-Aminopropyl)triethoxysilane, additionally all work must be done in a hood(f) |

| # Task (hands-on work or being within reach(a) of potential hazards of described activity/items): | Hazards: | Minimum Requirements: |
| --- | --- | --- |
| # IR Heatlamps # Inspecting samples under infrared (IR) heat lamps | Cataracts, flash burns to cornea | IR-blocking safety glasses |
| # Hot Liquids # Working with hot liquid samples on hotplates | Skin/eye damage | All work must be done in a hood(f); safety glasses, nitrile gloves, chemical-resistant apron, PVC arm shields Volume > 1 L: Add chemical-resistant overboots and coat apron |
| # Toxic Liquids # Working with toxic(d) inorganic liquids/mixtures # Beryllium compounds in solution | May be harmful if absorbed through skin. Causes skin irritation. May be fatal if absorbed through skin. Causes eye irritation. Toxic if swallowed. | Same as for Corrosive Liquids |
| # Beryllium Solids # Wet Hydroxide or wet/dry Nitrate (confined in quartz-glass vial) but not Beryllium Oxide | May be fatal if inhaled. Causes respiratory tract irritation. May be harmful if absorbed through skin. Causes skin irritation. May be fatal if absorbed through skin. Causes eye irritation. Toxic if swallowed. | REM hazardous solid Task Requirements: > 100 g any hazardous solid, or > 1 g "chemical requiring designated area," (list at REM web site(g)) or High potential for airborne particles Not applicable for WTHR 208; we handle micrograms of liquid beryllium and make ~1 milligram of solid beryllium salts at a time However, minimum requirements for any/all work in laboratory apply |
| # Beryllium Oxide # "Chemical requiring designated area," (list at REM web site(g)) and having a high potential for airborne particles | May be fatal if inhaled. Causes respiratory tract irritation. May be harmful if absorbed through skin. Causes skin irritation. May be fatal if absorbed through skin. Causes eye irritation. Toxic if swallowed. | Low-static vinyl gloves may be necessary when handling the oxide in quartz-glass vial. NIOSH N95 approved particulate mask for working with dry oxide or dry oxide and binder mix in an open vial |

| # Task (hands-on work or being within reach(a) of potential hazards of described activity/items): | Hazards: | Minimum Requirements: |
| --- | --- | --- |
| # Meker Burner or Propane Torch # Open High-Heat Flame | Skin/eye damage | Nitrile glove over cotton inspection glove to protect hand from heat or potential glove melting, safety glasses. Lab coat is NOT recommended unless needed to protect polyester clothing; only 100% Cotton lab coat should be used, no polyester content is allowed Tyvek |
| # AMS Standards # Preparing radioactive accelerator mass spectrometry standards (Activity in the pico- and femtoCurie range) | Cell damage, area contamination | All work must be done in the specific radioactive standards hood(f) ONLY; safety glasses, extended-cuff nitrile gloves, chemical-resistant apron. Discard gloves EACH time into the dedicated "Standards Trash" when you need to perform an activity outside of the standards hood. |
| # Accessing/Working at heights greater than eye level | Fall | Use portable ladder in the lab to reach shelves or perform other tasks that are higher than eye-level. Do NOT stand on chairs, stools or counters for these tasks. |

(a)	Being within reach of potential hazards: For Wetherill 208 "within reach" is defined as five (5) feet.

(b)	Unshielded: not behind a drawn hood sash or blast shield.

(c)	Corrosive: pH  12 or pH  2.5

(d)	Toxic: having any poisonous or irritating effects to human tissue or human health.

(e)	Chemical resistant gloves: glove thickness, length, and material must be chosen carefully and will be specific to the chemicals/mixtures used and the process conditions.

(f)	Hood: 100% exhaust to outside, current approval for "all work" and functioning properly.

(g)	Chemicals requiring designated areas: full list is at https://www.purdue.edu/ehps/rem/documents/programs/crdalist.pdf