### CERTIFICATION OF HAZARD ASSESSMENT 
Single Task

| Assessment Date(s): 2016-01-11 | Department: Physics & Astronomy |
| --- | --- |
| Building: | Room: |
| Supervisor (print): | Signature: |

| TASK OR ASSIGNMENT DESCRIPTION: | Use of HF (hydrofluoric acid) in laboratories run by PRIME Lab |
| --- | --- |

HAZARDS: HF is a highly corrosive acid which can severely burn skin, eyes, and mucous membranes.

Skin: Chemical burns. The fluoride ion readily penetrates the skin which can result in destruction of deep tissue layers.

Eyes: Contact with HF can result in destruction of the cornea.

Inhalation: Hydrofluoric acid fumes may cause spasmodic closure of—and accumulation of fluid in—the larynx, difficulty in breathing and/or accumulation of fluid in the lungs. Acute symptoms may include coughing, choking, chest tightness, chills, fever and skin turning blue.

Ingestion: Ingestion of HF may result in severe burns to the mouth, esophagus and stomach.

Burns larger than 25 square inches (160 square cm) may result in serious total body toxicity.

One of the most serious consequences of severe exposure to HF by any route is the marked lowering of calcium in the bloodstream, which may result in a fatal outcome if not recognized and treated.

| Task: hands-on work or being within reach(a) of potential hazards of described activity/items: | Minimum Requirements |
| --- | --- |
| All work with HF | All work must be performed in a fume hood unless otherwise noted No working alone unless you have specific authorization to do so Laboratory door must be unlocked Path to eyewash/safety shower must be unblocked 2.5% calcium gluconate gel location: _________________ 1% calcium gluconate eyewash location: ______________ Milk of Magnesia bottle location: _____________________ Have water running in an external sink to rinse gloves |
| Dispensing HF >40% (by weight) from its originating container via pouring or approved spout Volumes > 5 ml | Safety glasses/goggles AND full-face shield Vinyl coat apron (to cover your shoulders) Neoprene apron (to cover your front) Chemical resistant gloves: natural rubber, nitrile, or vinyl Nitrile examination gloves can be doubled over the rubber gloves for grip, if necessary PVC arm shields Chemical resistant overboots |
| Manipulating open digestion vessels when the original concentration of HF has been diluted with water and/or other reagents: Volumes < 5 ml | Safety glasses/goggles Neoprene apron Nitrile examination gloves PVC arm shields |
| Manipulating open vessels (e.g. centrifuge tubes) when the original concentration of HF has been diluted to ≤5% All Volumes | Work in fume hood not necessary Safety glasses/goggles Neoprene apron Nitrile/latex examination gloves |

(a)	Being within reach of potential hazards: for PRIME Lab areas "within reach" is defined as five (5) feet.