### CERTIFICATION OF HAZARD ASSESSMENT 
Single Task

| Assessment Date(s): 2013-08-09 | Department: Physics |
| --- | --- |
| Building: Wetherill / Physics | Room: WTHR 206 WTHR 208 PHYS B174 PHYS B23 PHYS S187 |
| Supervisor (print): George Jackson | Signature: |

| TASK OR ASSIGNMENT DESCRIPTION: | Use of HF (hydrofluoric acid) in laboratories run by PRIME Lab |
| --- | --- |

HAZARDS: HF is a highly corrosive acid which can severely burn skin, eyes, and mucous membranes.

Skin: Chemical burns. The fluoride ion readily penetrates the skin which can result in destruction of deep tissue layers.

Eyes: Contact with HF can result in destruction or opacification of the cornea.†

Inhalation: Hydrofluoric acid fumes may cause laryngospasm, laryngeal edema, bronchospasm and/or acute or delayed pulmonary edema. Acute symptoms may include coughing, choking, chest tightness, chills, fever and cyanosis.

Ingestion: Ingestion of HF may result in severe burns to the mouth, esophagus and stomach.

Burns larger than 25 square inches (160 square cm) may result in serious systemic toxicity.

25 Square Inches

One of the most serious consequences of severe exposure to HF by any route is the marked lowering of serum calcium (hypocalcemia) and other metabolic changes, which may result in a fatal outcome if not recognized and treated.

While hypocalcemia has been traditionally considered the major systemic effect of severe poisoning with HF, it is apparent that hypomagnesemia, hyperkalemia, the cardiodepressing and vasodilating effects of fluoride and effects on pulmonary hemodynamics and systemic capacitance vessels, including an increase in pulmonary vascular resistance, all play a role in systemic toxicity

† See the HF Binder for definitions of medical terms.

| Task: hands-on work or being within reach(a) of potential hazards of described activity/items: | Minimum Requirements (Task/Requirements for this laboratory are bold-bordered.) |
| --- | --- |
| All work with HF | All work must be performed in a fume hood unless otherwise noted No working alone unless you have specific authorization to do so Laboratory door must be unlocked Path to eyewash/safety shower must be unblocked 2.5% calcium gluconate gel must be present Have water running in an external sink to rinse gloves |
| Dispensing HF >40% (by weight) from its originating container via pouring | Safety glasses/goggles AND full-face shield Vinyl coat apron (to cover your shoulders) Neoprene apron (to cover your front) Chemical resistant gloves: natural rubber, nitrile, or vinyl Nitrile examination gloves can be doubled over the rubber gloves for grip, if necessary PVC arm shields Chemical resistant overboots |
| Dispensing HF >40% (by weight) from its originating container via approved spout Volumes > 5 ml | Safety glasses/goggles AND full-face shield Neoprene apron Chemical resistant gloves: natural rubber, nitrile, or vinyl Nitrile examination gloves can be doubled over the rubber gloves for grip, if necessary PVC arm shields Chemical resistant overboots |
| Dispensing HF >40% (by weight) from its originating container via approved spout Volumes < 5 ml | Safety glasses/goggles Neoprene apron Chemical resistant gloves: natural rubber, nitrile, or vinyl PVC arm shields |
| Manipulating HF in sealed digestion vessels when the original concentration has been diluted with water and/or other reagents: Volumes > 5 ml | Work in fume hood not necessary Safety glasses/goggles Neoprene apron Chemical resistant gloves: natural rubber, nitrile, or vinyl PVC arm shields Chemical resistant overboots |
| Manipulating HF in open digestion vessels when the original concentration has been diluted with water and/or other reagents: Volumes > 5 ml | Safety glasses/goggles AND full-face shield Neoprene apron Chemical resistant gloves: natural rubber, nitrile, or vinyl Nitrile examination gloves can be doubled over the rubber gloves for grip, if necessary PVC arm shields Chemical resistant overboots |
| Manipulating HF in open digestion vessels when the original concentration has been diluted with water and/or other reagents: Volumes < 5 ml | Safety glasses/goggles Neoprene apron Nitrile examination gloves PVC arm shields |
| Manipulating HF in open vessels (e.g. centrifuge tubes) when the original concentration has been diluted to ≤5% All Volumes | Work in fume hood not necessary Safety glasses/goggles Neoprene apron Nitrile/latex examination gloves |