PHYS 342L

NOTES ON ANALYZING DATA

Spring Semester 2002

Department of Physics

Purdue University

A major aspect of experimental physics (and science in general) is measurement  of some quantities and analysis of experimentally obtained data. While there are a lot of books devoted to this problem, in the next paragraphs we will summarize some of the important ideas that will be needed to successfully analyze data acquired in PHYS 342L. Students are advised to consult with [1] for more detailed discussion on the topic.

1. The importance of estimating errors.

Suppose you are asked to measure the length of a piece of notebook paper. You grab a ruler and proceed with a measurement. The ruler shows 276 mm. Does it mean the length is 276.0000 mm? Most probably not. Why? Because the distance between the neighboring marks on your ruler is 1 mm, by saying 276 mm you cannot exclude, for example, length 276.2 or 259.9 mm. Thus you assume certain precision (or error) in your measurement, in this case it is probably ~0.5 mm, as the distance between the closest marks is 1 mm. The result of the meausurement is not just the length of the paper but also the error of this measurement: (276.00.5) mm. In a scientific experiment, both parts of measurement are important. Suppose you measure the length of the next sheet of paper to be (275.50.5) mm. Within the error of your measurement these two sheets of paper have the same length.

2. Precision (or Accuracy) of a Measurement.

Distinguish between absolute uncertainty and relative uncertainty:

| absolute uncertainty | relative uncertainty |
| --- | --- |
| 27.6  0.1 | ±0.127.6=±0.003623188 |

All these numbers don’t mean much when calculating the relative uncertainty, so round off to  0.004, or, expressed as a percent,  0.4%.

3. Combining Uncertainties.

Suppose that you measure two quantities A and B. Suppose you measure A to an accuracy of A and B to an accuracy of B.

How do you algebraically combine these uncertainties?

a) When adding:

(A  A) + (B  B) = ?

there are four possibilities:

(A + A) + (B + B) = (A + B) + (A + B)

(A + A) + (B - B) = (A + B) + (A - B)

(A - A) + (B + B) = (A + B) - (A - B)

(A - A) + (B - B)  = (A + B) - (A + B)

clearly, the worst case will be (A+B)(A+B)					(1)

b) When subtracting:

(AA) - (BB) =?

Again consider four cases.  From above, it should be obvious that the worst case will be given by

(A-B)(A+B)						(2)

c) When multiplying

(A±δA)×(B±δB)=AB+A(±δB)+B(±δA)+(±δA)(±δB)small,neglectAB±(AδB+BδA)AB1±δBB+δAA			(3)

d) When dividing

A±δAB±δB=?

After some algebra, you find that

A±δAB±δB≈AB1±δAA+δBB 				(4)

Remember:

- relative uncertainties add when multiplying or dividing.

- absolute uncertainties add when adding or subtracting

4. Systematic and random errors.

Figure 1:  Spread in the measurement of some quantity x in the absence of systematic error (left) and in its presence (right).

If error in your measurements is random, then the average value should be close to the actual value. In the case of systematic error, that is not true. This situation may occur when, for example, using a clock which is running slow to measure some time period. Random errors are inevitable, while systematic errors can be taken into account or eliminated.

5. Average value and standard deviation.

In order to decrease the influence of random error multiple measurements xi are taken and averaged:

x=1ni=1nxi					(5)

How close this average value x is to the actual value X? If we have a set of measurements we can find an average error for a single measurement. The commonly accepted value to characterize error is called standard deviation , or root mean square (rms):

σ2=1ni=1nxi−X2				(6)

Since the actual value X is usually unkown, we must use x instead. It can be shown [1] that in this case:

σ2≈1n−1i=1nxi−x2			(7)

The value  characterizes error in a single measurement of value X. If we take several measurements of the same value x and average them, the resulting value x must in average be closer to actual value x as a single measurement. It can be shown [1] that standard deviation n for the average value of n measurements is:

σn=σn					(8)

###### 6. Distribution of measurements

A series of  measurements may be represented as a histogram (Fig. 2).

Figure 2. A simple histogram after taking just five data points (n=5). There was only one data point falling into range of x marked as A, B and D, and two measutrements where in region C.

It is difficult to see any trends after taking just a few data points.  Make more measurements and use smaller bins and you’ll eventually get a histogram that might look like this.

Figure 3. An histogram after taking hundreds of measurements.

In a limit of large n the distribution is given by continuous distributin function f(x), so that f(x)dx is the probability that a single measurement taken at random will lie in the interval x to x+dx. The average value can be then found as:

x=−∞∞x⋅f(x)dx				(9)

And standard deviation:

σ2=x2=−∞∞(x−X)2f(x)dx≈−∞∞x−x2f(x)dx		(10)

In many cases error distribution function is well described by Gaussian (also called normal distribution (Fig. 4):

f(x)=1σ2πe−x−X22σ2				(11)

Figure 4. Gaussian distribution function.

The standard deviation  for Gaussian distribution can be also expressed as:

σ=fwhm22ln(2)≈0.425×fwhm		(12)

where fwhm if full width at half maximum, which can be estimated graphically.

Suppose now that we performeed a single measurement which resulted in value x and we also know the standard deviation of this measurement . Since the Gaussian distribution is a continuous function which becomes zero only in infinity, the measured value x may lay anywhere from - to +.  What is the probability that the actual value X which we are trying to measure is within distance  from this measured value? Since f(x)dx is the probability of measuring value between x and x+dx, the probability of measuring x between X- to X+ is given by integralX−σX+σf(x)dx=0.68.

Thus, the probablity of your measurement x being within

X 	- 68%

X2 	- 95%

X3 	- 99.7%

X4 	- 99.994%

6. Combining ’s.

Let us now get back to the case when we add two values A and B, where standard deviations are A and B, correspondingly. What would be the standard deviation of the sum A+B? As we already know, the errors should be added for the worst case scenario (Eq. 1). However, if errors in A and B are random and mutually uncorrelated, they tend to cancel to some extent as there is 50% probability that they have different sign in one measurement set. It can be shown, that the standard deviation of the sum (or difference) is:

σA±B=σA2+σB2				(13)

Notice that ABA+B. If A=B=, then σA±B=σ2.

Similarily, the relative standard deviation C/C for product (or ratio) of A and B is:

σCC=σAA2+σBB2			(14)

where C=AB or C=A/B.

Note, that this is true only if errors are uncorrelated and not systematic.

7. A simple example

Suppose you need to evaluate the charge to mass ratio of an electron.  This is to be done using the following equations

qm=2VB2R2, 				(15)

where  B=kI

How would you analyze the error in qm?

Suppose you measure I, V and k as follows:

I=(1.40.1) A

V=(1402) V

k=coil constant=(7.50.5)10-4T/A

You also measure R, the radius of the electron’s orbit, by measuring its diameter D. Since the smallest marking on the ruler is 1 mm and you have to determine the positions of both sides of the electron orbit, the precision of such a measurement is not better than 2mm = 0.2 cm. To reduce random error you may want to take several measurements. Suppose you make three measurements of D:

D1=6.0 0.2 cm	R1=D1/2=3.00.1 cm

D2=5.80.2 cm	R2=2.90.1 cm

D3=5.70.2 cm	R3=2.850.1 cm

Using Eqs. 5 and 7, the average value of R and the standard deviation  for one measurment  will be

R=3.0+2.9+2.853=2.92(cm)

σ=0.082+0.022+0.0722=0.054

According to Eq. 8, for the average of 3 measurements the standard deviation 3 will be

σ3=0.0543≈0.03

However, the ruler we use has precision ~0.1 cm only. Using Eq. 13 we can account for both errors:

ΔR=σ32+0.12≈0.11cm

and we have

R= (2.920.11) cm

Now we calculate:

qm=2VB2R2=2Vk2I2R2qm=2(140±2)(7.5±0.5)×10−421.4±0.120.0292±0.00112	(16)

Omitting errors we get the value of qm=2.98×10−11Ckg

Using Eq. 14 we can write:

σqnqn=σVV2+2σkk2+2σII2+2σRR2		(17)

Note the factors “2” in the equation above. These stem from the fact that corresponding values are squared in Eq. 16, i.e. we have products kk , II and RR. Since kk is a product of two correlated values, we must use Eq. 3 – the relative errors simply add up.

σqn=2.98×10−1121402+2×7.50.52+2×0.11.42+2×0.00110.02922σqn=0.63×10−11Ckg

And the final result for the ratio can be written as:

qm=(2.98±0.63)×10−11Ckg

Here we used Eq. 14 since values V, k, I and R are uncorrelated. If we assume, just for example, a correlated case, we must use Eq. 3 and 4 – i.e. add relative errors:

σqnqn=σVV+2σkk+2σII+2σRR

and	σqn=1.10×10−11Ckg

The accepted value is 1.761011 C/kg.  It’s easy to make a simple plot including error bars to graphically illustrate this result.

Figure 6:   A plot showing the measured value with error bar.  The dotted horizontal line represents the accepted value.  The q/m axis has units of 1011 C/kg.

The measured value of q/m is ~2 higher than the accepted value. The probability for that to happen is only ~5%, which strongly suggests the presence of some systematic error in our measurement.

8. Least Squares Fit.

Suppose you measure some data points y as a function of a variable called x. After the measurements, you will have a set of data points

x1, y1

x2, y2

……

xN, yN

Sometimes you might know that the data should fit a straight line (e.g., from theoretical considerations). The equation of a straight line is

y=mx+b

where the slope m might equal ‘a certain quantity of interest’ and the intercept b might equal ‘some other quantity of interest’.

Figure 7:   A plot showing the best straight line fit to a collection of data points.

In short, if you could determine m and b, these values may contain estimates for useful quantities.  One way to determine m and b is to plot the data and use a ruler to draw a straight line through the points.  Then, by calculating m and b from the straight line drawn, you have produced some weighted average estimate of m and b from all your data.

A simple example?  Suppose you are asked to determine  experimentally and suppose you already know that for circles

circumference =  (diameter)

One way to proceed might be to make a variety of circles of different diameters and then measure the circumference of each one.  You might plot the data as follows:

Figure 8:   A plot of how the measured values for the circumference of different circles might vary as a function of the measured diameter.  Note that in this example, the intercept of the best straight line through the data MUST pass through the co-ordinate origin.

Clearly, the slope of a straight line through the data contains useful information since =slope.

Q: How can you determine the ‘best value’ for the slope and intercept without prejudice or personal judgment?

A: Use the principle of least squares.

Assume you draw N circles and make measurements of each circumference and diameter. Let the independent variable (the diameter) be represented by the symbol d.  Let the dependent variable (the circumference) be represented by the symbol C. Also assume the d values are accurate.  After the measurement process, you’ll have a set of numbers (d,C):

d1, C1

d2, C2

……..

dN, CN

It is conventional to map these numbers into the parameters (x,y) as follows

x1=d1, y1= C1

x2=d2, y2= C2

……..

xN=dN, yN= CN

Let the difference between the ‘best line’ through the data and each individual data point be represented by (yi). One unambiguous way to specify the ‘best line’ through all the data can be defined by the condition that the sum of all the (yi)2 have a minimum value.

How are the individual yi defined?  Graphically, they are indicated in the plot below.  Note that at this point of the analysis, the straight line drawn need not be the best straight line through the data.

Figure 9. A least squares analysis requires you to calculate the deviation of each data point from the ‘best’ straight line.

Mathematically, you can calculate the yi as follows. Suppose you define a quantity Yi such that Yi=mxi+b where the symbols m and b are somehow chosen to represent the ‘best’ straight line through the data, whatever that means.  Calculate

yi=yi-Yi=Ci-(mdi+b)

Least Squares Fitting requires that (where the switch in notation from (C,d) to (y,x) has been made)

i=1Nδyi2=i=1Nyi−mxi+b2=minimum

Write  [yi-(mxi+b)]2=M. The conditions for M to be a minimum are

∂M∂m=0,      ∂M∂b=0

Performing the derivatives and setting them equal to zero gives, after some algebra, two unique equations for the ‘best’ m and b.

m=Ni=1N(xiyi)−i=1Nxii=1NyiNi=1Nxi2−i=1Nxi2

m=i=1Nxi2i=1Nyi−i=1Nxi(xiyi)Ni=1Nxi2−i=1Nxi2

Once we have m and b, then also calculate the intermediate quantity y:

σy=i=1Nyi−mxi+b2N−2

It can be shown that the uncertainty in the slope m and intercept b is given by

σm=σyNNxi2−xi2

σm=σyxi2Nxi2−xi2

You can conclude that the best values for m and b are

mm     bb

This means that there is a 68% chance of the real m lying between m-m and m+m. Likewise, there is a 68% chance of the real b lying between b-b and b+b.

Since the least squares fitting formulae involve sums over various combinations of measured data, the least squares fitting procedure is especially easy to implement in spread sheets like Excel.  In fact, most spread sheet programs have pre-programmed least square fit routines available as analysis tools.

Reference

1. G. L. Squires. Practical Physics, 4th Edition, Cambridge University Press, Cambridge (2001)